Milk Paint

1 PINT				1 Quart

Wooden Craft Sticks

Lidded Plastic Jars 4 oz

Craft Brushes

Strainer

Microplane

0000 Steel Wool

Wipe-On Poly

Walnut Oil & Bees Wax

Kroger has in stock			On Line

Mixture is 1 part Bees Wax & 4 parts Walnut Oil.

- In a double boiler, heat up pellets of Bees Wax until  liquified

- Add Walnut Oil to liquified bees wax and mix thoroughly

- Pour into suitable container and let is cool.

- Apply paste to raw wood or burnished Milk Paint using a paper towel with lathe off

- Once the surface is coated with past turn on lathe to relatively high speed and work paste into the turning.

- Take a clean paper towel and increase speed to 1500 rpm and buff to a nice luster.